PSI Recommendation

PSI Mass Spectrometry and Proteomics Informatics Working Groups

Status: DRAFT

Eric W. Deutsch, Institute for Systems Biology

Joshua Klein, Boston University

Hans Vissers, Waters Corporation

Hannes Röst, University of Toronto

Matt Chambers, Consultant

Tim Van Den Bossche, Ghent University

Tomi Suomi, University of Turku

Yasset Perez-Riverol, European Bioinformatics Institute (EMBL-EBI)

Andrew Dowsey, University of Bristol

Pierre-Alain Binz, Centre Hospitalier Universitaire Vaudois

Recommendation for encoding data independent acquisition, ion mobility data, subsampled data arrays, and additional compression types in mzML 1.1

Status of this document

This document provides information to the proteomics community about a proposed recommendation for encoding additional information in the existing mzML 1.1 format. Distribution is unlimited.

Version: Draft 97 of Version 1.0

Abstract

The Human Proteome Organisation (HUPO) Proteomics Standards Initiative (PSI) defines community standards for data representation in proteomics to facilitate data comparison, exchange and verification. This document presents a consensus community practice description on how to encode the data from various data independent analysis (DIA) workflows, including scanning quadrupole data independent acquisition (e.g., SONAR) and ion mobility assisted data independent acquisition workflows (e.g., HDMSE, and diaPASEF), subsampled data arrays, and additional compression types in mzML 1.1 in the PSI standard format mzML 1.1. It was consensus that the data could be encoded with only the addition of controlled vocabulary terms (Mayer et al. 2013) and the additional guidance provided in this document, and no schema change was required.

Contents

1.	Introduction	2

1.1	Description of the need	2

1.2	Requirements	2

1.3	Issues to be addressed	3

2.	Notational Conventions	3

3.	The Format Implementation	3

3.1	The documentation	3

3.2	Relationship to other specifications	3

3.3	Adjustments for raw DIA data such as SONAR, HDMSE, or diaPASEF data	3

3.4	Adjustments for deconvoluted DIA data such as SONAR or HDMSE data	4

3.5	Adjustments for differentiating between low and high energy scans	6

3.6	Adjustments for front-end ion mobility filtering	6

3.7	Parallel subsampled data arrays	7

3.8	Additional compression types	7

4.	Authors Information	7

5.	Contributors	8

6.	Intellectual Property Statement	8

7.	Copyright Notice	9

8.	Glossary	9

9.	References	9

# Introduction

## Description of the need

The mzML format (Martens et al. 2011) is an open, comprehensive XML-based format for encoding mass spectrometer output data. It is the only such format approved by the Human Proteome Organization (HUPO) Proteomics Standards Initiative (PSI) (https://psidev.info/). The format has remained stable and unchanged since the mzML 1.1 release in June 2009.

Several advances in instrumentation, mass spectrometry experiments and data processing need to be represented adequately in mzML. This document is a collection of additional recommendations how data SHOULD be encoded in mzML 1.1 by specifying a) additional requirements for the presence and usage of cvTerms and b) recommendations for (additional) data arrays to capture raw and preprocessed data with the aim to minimize differences in implementation amongst software packages that can properly handle these types of data.

Recently developed workflows by different instrument vendors include Waters Corporation SONAR (scanning quadrupole data independent acquisition) (Moseley et al. 2018) and HDMSE (ion mobility assisted data independent acquisition) (Rodriguez-Suarez et al. 2013), and Parallel Accumulation – Serial Fragmentation combined with data-independent acquisition (diaPASEF) by Bruker Daltonik (Meier et al. 2019). It has become important to clarify how data from these workflows SHOULD be encoded in the mzML 1.1 format to minimize differences in implementation amongst software packages that can properly handle these workflows. There are two primary forms of data from these two workflows: the original high dimensional data and the reduced-dimensionality deconvoluted data. Although the primary driver of these requirements has been from Waters Corporation and Bruker Daltonik, it is desirable to describe these new technologies in somewhat generic terms that can apply to similar technologies from other vendors. File sizes for DIA and IMS workflow acquisitions can become very large, and a binary version of mzML, called mzMLb (Bhamber et al. 2021), can lead to smaller files.

## Requirements

The main requirements to be fulfilled are:

- Provide a clear single recommended way to encode and interpret the raw output data from SONAR, HDMSE, diaPASEF and similar data-independent acquisition (DIA) workflows in mzML 1.1.

- Provide a clear single recommended way to encode and interpret the deconvoluted data from SONAR, HDMSE, and similar DIA workflows in mzML 1.1.

- Provide a clear single recommended way to encode and interpret Ion Mobility data.

- Encode the aforementioned information without changing the XML schema.

- Provide guidance on additional binary data arrays that may not parallel the coordinate axis, including sampled arrays.

- Recommend additional compression methods to further reduce file sizes and/or speed up encode/decoding, including combining lossless and lossy methods together.

## Issues to be addressed

The main issues to be addressed by this recommendation are:

- The only issue to be addressed is the potential confusion or ambiguity on how the aforementioned data types are encoded in mzML 1.1.

# Notational Conventions

The key words “MUST“, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” are to be interpreted as described in RFC 2119 (Bradner 1997).

Essentially, “MUST” denotes REQUIRED, “SHOULD” denotes recommended, “MAY” denotes optional.

# The Format Implementation

## The documentation

All information for this best practice recommendation is provided in this document. Potential updates to this document or other follow-on recommendations for other workflows are provided at the mzML documentation page at the HUPO-PSI website (http://www.psidev.info/mzML).

## Relationship to other specifications

This recommendation is a direct addendum to the main mzML 1.1 specification, which can be found at http://www.psidev.info/mzML.

## Adjustments for raw DIA data such as SONAR, HDMSE, or diaPASEF data

When raw (i.e. not deconvoluted) data from SONAR, HDMSE, diaPASEF or similar workflows are encoded in mzML, the file content header SHOULD contain one or more of the following merged-concept CV terms (Table 1), depending on the appropriate workflow. Individual constituent terms are also defined in the CV, but SHOULD NOT be used. Example datasets used as part of ProteoWizard’s test suite are included for each method in Table 2. Additional data arrays may also be stored next to the m/z and intensity arrays to encode additional raw or preprocessed data. This lets us preserve information that would otherwise be lost during translation to mzML from the vendor format, or during intermediate processing in one stage of the data processing pipeline, and fosters more consistent interfaces for software that can properly handle these types of data.

For SONAR or other scanning quadrupole data, the lower bound and upper bound of the scanning quadrupole isolation window MUST be provided using two additional data arrays next to the m/z array and intensity arrays as follows:

<cvParam cvRef="MS" accession="MS:1003157" name="scanning quadrupole position lower bound m/z array"

unitAccession="MS:1000040" unitName="m/z" unitCvRef="MS" />

and

<cvParam cvRef="MS" accession="MS:1003158" name="scanning quadrupole position upper bound m/z array"

unitAccession="MS:1000040" unitName="m/z" unitCvRef="MS" />

Table 1. Merged-concept CV terms for raw DIA data.

| Workflow | CV terms to use | Constituent terms |
| --- | --- | --- |
| MSE AIF p2cid | - data independent acquisition from dissociation of full mass range (MS:1003227) | - data-independent acquisition (MS:1003215) - dissociation of full mass range (MS:1003216) |
| SONAR | - data independent acquisition from dissociation of scanning quadrupole across mass range (MS:1003228) | - data-independent acquisition (MS:1003215) - dissociation of scanning quadrupole across a specified mass range (MS:1003217) |
| SWATH-MS FT-ARM HRM PAcIFIC | - data independent acquisition from dissociation of sequential mass ranges (MS:1003224) | - data-independent acquisition (MS:1003215) - dissociation of sequential mass ranges (MS:1003218) |
| diaPASEF | - data independent acquisition from dissociation of sequential mass ranges after ion mobility separation (MS:1003225) | - data-independent acquisition (MS:1003215) - dissociation of sequential mass ranges (MS:1003218) - ion mobility separation |
| HDMSE IMS-AIF | - data independent acquisition from dissociation of full mass range after ion mobility separation (MS:1003226) | - data-independent acquisition (MS:1003215) - dissociation of full mass range (MS:1003216) - ion mobility separation (MS:1003219) |

If additional updates to this table are necessary in the future, this information will be revised in this table in the mzML GitHub repository: https://github.com/HUPO-PSI/mzML/tree/master/specification_document/dia_workflow_cv.md.

Raw MS1 and MS2 scans are encoded according to the original mzML 1.1 specification.

However, for ion mobility data, instead of encoding each ion mobility frame to a different <spectrum>, which leads to millions of spectra per run, it is recommended that each sweep of ion mobility is recorded as a single <spectrum> with an extra ion mobility dimension. In addition to the “m/z array” and “intensity array” (and potentially other arrays) that make up each spectrum, an additional parallel array SHOULD be provided with the most appropriate child term of “ion mobility array” (MS:1002893), depending on the units and degree of processing, e.g. “raw inverse reduced ion mobility array” (MS:1003008) or “deconvoluted ion mobility drift time array” (MS:1003156). The ambiguous parent term “ion mobility array” (MS:1002893) itself MUST NOT be used in a data file.

There was some concern that in the original mzML 1.1 format there should never be two identical m/z values in an “m/z array”. But this was not explicitly stipulated in the mzML 1.1 specification, and with the addition of ion mobility arrays, repetition in the “m/z array” will be common (having differing ion mobility array values). This means that all mzML parsers SHOULD be checked/updated to handle mzML data with non-increasing (identical or decreasing) m/z values in the "m/z array". All parsers SHOULD also be updated to warn or halt gracefully if they are not prepared to handle the kinds of data described herein.

The set of binary arrays SHOULD be sorted in order of ascending m/z, with the ion mobility value (any type) in ascending order as the second sorting criterion if it is present. It is expected that this order will compress most efficiently. However, this sort order is not enforced by the mzML validator. Software SHOULD be prepared to handle other orders.

Table 2. Example datasets

| Acquisition method | URL |
| --- | --- |
| MS:1003225 diaPASEF | https://github.com/ProteoWizard/pwiz/tree/master/pwiz/data/vendor_readers/Bruker/Reader_Bruker_Test.data/diaPASEF.d |
| MS:1003228 SONAR | https://github.com/ProteoWizard/pwiz/tree/master/pwiz/data/vendor_readers/Waters/Reader_Waters_Test.data/SONAR_Short.raw |
| MS:1003226 HDMSE | https://github.com/ProteoWizard/pwiz/tree/master/pwiz/data/vendor_readers/Waters/Reader_Waters_Test.data/HDMSe_Short_noLM.raw |
| MS:1003227 MSE | https://github.com/ProteoWizard/pwiz/tree/master/pwiz/data/vendor_readers/Waters/Reader_Waters_Test.data/MSe_Short.raw |
| MS:1003224 SWATH-MS | https://github.com/ProteoWizard/pwiz/blob/master/pwiz/data/vendor_readers/ABI/Reader_ABI_Test.data/swath.api.wiff2 |

## Adjustments for deconvoluted DIA data such as SONAR or HDMSE data

We define deconvoluted data as average properties of an analyte post peak-detection, weighted charge state reduction, and/or adduct aggregation. When deconvoluted data from SONAR, HDMSE, or similar workflows are encoded in mzML, the file content header SHOULD contain one or more of the merged-concept CV terms from Table 1 and the term “deconvoluted data” (MS:1003398) depending on the appropriate workflow. As in section 3.3, the constituent terms should not be used.

Furthermore, the <selectedIonList> element MUST contain a list of all of the measured analyte sibling ions (i.e. different charge states of the same molecule) that are associated with the fragment ion spectrum. This is in contrast to mzML from other workflows where the entries in the <selectedIonList> are one or more unrelated analyte ions that happened to be included in the isolation window. For example:

<selectedIonList count="2">

<selectedIon>

<cvParam cvRef="MS" accession="MS:1000744" name="selected ion m/z" value="666.679163727272" unitCvRef="MS" unitAccession="MS:1000040" unitName="m/z"/>

<cvParam cvRef="MS" accession="MS:1000041" name="charge state" value="2"/>

<cvParam cvRef="MS" accession="MS:1000042" name="peak intensity" value="0.87e05" unitCvRef="MS" unitAccession="MS:1000131" unitName="number of detector counts"/>

</selectedIon>

<selectedIon>

<cvParam cvRef="MS" accession="MS:1000744" name="selected ion m/z" value="333.679163727272" unitCvRef="MS" unitAccession="MS:1000040" unitName="m/z"/>

<cvParam cvRef="MS" accession="MS:1000041" name="charge state" value="3"/>

<cvParam cvRef="MS" accession="MS:1000042" name="peak intensity" value="0.40e05" unitCvRef="MS" unitAccession="MS:1000131" unitName="number of detector counts"/>

</selectedIon>

</selectedIonList>

Here, both selected ions are 2+ and 3+ siblings of the same analyte from which the deconvoluted fragment ion spectrum is created. Software reading this information MAY display a single analyte where the charge-reduced precursor mass (M+H)+ and total ion intensity have been computed from the above information, with an option to see the individual ions.

Moreover, for any DIA acquisition of a full mass range (such as MSE or HDMSE data), the <isolationWindow> element SHOULD contain the marker MS:1003159 “no isolation” term  :

<cvParam cvRef="MS" accession="MS:1003159" name="no isolation"/>

Note that the <isolationWindow> element is required by the current schema and this requirement is maintained, but is valid containing just MS:1003159 marker term, so there is no formal requirement to fill the element with placeholder information. Compliant readers SHOULD be able to tolerate these terms being absent.

## Adjustments for differentiating between low and high energy scans

There has been substantial confusion in the community about the best way to differentiate between the low energy and high energy scans from e.g. Waters Corporation instruments for MSE, HDMSE, and SONAR acquisitions. Several possible strategies were proposed, but the selected implementation requires no schema change and is as follows.

Low-energy scans will be encoded as MS Level 1 scans. High-energy scans will be encoded as MS Level 2 scans with an <isolationWindow> as described in section 3.4 and the collision energy applied encoded as usual.

## Adjustments for front-end ion mobility filtering

Some instruments support filtering of ions entering a mass spectrometer by ion mobility. A common mechanism is the high-field asymmetric waveform ion mobility spectrometry (FAIMS) device, for which each scan may have a single compensation voltage value. If FAIMS information is available it MUST be encoded in mzML files by specifying the “FAIMS compensation voltage” CV term (MS:1001581) in the <scan> element. For example:

<scan instrumentConfigurationRef="IC1">

<cvParam cvRef="MS" accession="MS:1000016" value="6.437239112" name="scan start time"

unitAccession="UO:0000031" unitName="minute" unitCvRef="UO" />

<cvParam cvRef="MS" accession="MS:1000927" value="118" name="ion injection time"

unitAccession="UO:0000028" unitName="millisecond" unitCvRef="UO" />

<cvParam cvRef="MS" accession="MS:1001581" name="FAIMS compensation voltage" value="-54.0"

unitAccession="UO:0000218" unitName="volt" unitCvRef="UO" />

Note that some converters (e.g. msconvert) prior to 2021-04-01 placed the “FAIMS compensation voltage” term in the <spectrum> element. Although this is now deprecated, readers MAY also consult the <spectrum> element to see if the FAIMS compensation voltage term is present there.

A similar technology is the SelexION differential mobility separation from SCIEX. For data acquired with the SelexION device, if the device voltage is available, the information MUST be encoded in mzML files by specifying the “SelexION separation voltage” (MS:1003394) CV term and the “SelexION compensation voltage” (MS:1003371)  in the <scan> element. For example:

<scan instrumentConfigurationRef="IC1">

<cvParam cvRef="MS" accession="MS:1000016" value="12.39486" name="scan start time"

unitAccession="UO:0000031" unitName="minute" unitCvRef="UO" />

<cvParam cvRef="MS" accession="MS:1000927" value="60" name="ion injection time"

unitAccession="UO:0000028" unitName="millisecond" unitCvRef="UO" />

<cvParam cvRef="MS" accession="MS:1003394" name="SelexION separation voltage" value="124.0"

unitAccession="UO:0000218" unitName="volt" unitCvRef="UO" />

<cvParam cvRef="MS" accession="MS:1003371" name="SelexION compensation voltage" value="20.0" />

## Parallel subsampled data arrays

In a standard mzML <spectrum><binaryDataArrayList>, each of the data arrays SHOULD have the same number of elements. However, there is a requested use case to allow a second parallel array that is sampled with fewer m/z data points. The requested use case is for storing subsampled (i.e. interpolated to fewer data points) noise information. In order to support such a use case, the following set of “sampled” data arrays are available in the CV for use here:

id: MS:1002743

name: sampled noise m/z array

def: "A data array of parallel, independent m/z values for a sampling of noise across a spectrum (typically much smaller than MS:1000514, the m/z array)." [PSI:MS]

id: MS:1002744

name: sampled noise intensity array

def: "A data array of intensity values for the amplitude of noise variation superposed on the baseline (MS:1002745) across a spectrum (for use with MS:0002743, sampled noise m/z array)." [PSI:MS]

id: MS:1002745

name: sampled noise baseline array

def: "A data array of baseline intensity values (the intensity in the absence of analytes) for a sampling of noise across a spectrum (for use with MS:0002743, sampled noise m/z array)." [PSI:MS]

## Additional compression types

At the original release of mzML 1.1, only “zlib compression” and “no compression” were supported compression types. The addition of the widely supported MS-Numpress lossy compression codecs expanded the field, but these may now also be combined with zlib:

(MS:1000576) no compression

(MS:1000574) zlib compression

(MS:1002312) MS-Numpress linear prediction compression

(MS:1002746) MS-Numpress linear prediction compression followed by zlib compression

(MS:1002313) MS-Numpress positive integer compression

(MS:1002747) MS-Numpress positive integer compression followed by zlib compression

(MS:1002314) MS-Numpress short logged float compression

(MS:1002748) MS-Numpress short logged float compression followed by zlib compression

(MS:1003088) truncation and zlib compression

(MS:1003089) truncation, delta prediction and zlib compression

(MS:1003090) truncation, linear prediction and zlib compression

along with 2 additional terms that can describe settable parameters:

(MS:1003091) binary data compression parameter

(MS:1003092) number of mantissa bits truncated

These compression types have been implemented in ProteoWizard for reading and writing, and other software tools may encounter files written with these compression techniques.

In addition to zlib compression, we also recommend the zstandard (zstd) compression method. zlib was invented in 1995, and has been constrained by the need for backwards compatibility, and while it is still an efficient compressor, even its highly optimized implementations are slow compared to more modern lossless compression algorithms. Zstandard is a very fast compressor and often yields compression as good as zlib, if not better. Additionally, techniques for losslessly making data more compressible have become more common. To that end we recommend the following additional compression methods:

(MS:1003780) zstd compression

(MS:1003781) byte-shuffled zstd compression

(MS:1003782) dictionary-encoded zstd compression

(MS:1003783) MS-Numpress linear prediction compression followed by zstd compression

(MS:1003784) MS-Numpress positive integer compression followed by zstd compression

(MS:1003785) MS-Numpress short logged float compression followed by zstd compression

## Compression with byte shuffling

For some numerical data, it is possible to achieve (much) better compression performance by re-arranging the bytes (or bits) of the elements of the data array prior to compressing them. This is due to how numbers are laid out in memory, and shuffling may not always improve compression ratios. Also, as a note to some readers, while we use the term “shuffle”, this is not a random operation analogous to shuffling a deck of cards is supposed to be, but closer in execution to a structured re-arrangement of bytes in multi-byte data types. Consider the following array of two numbers:

[549.1  552.22]

We can view these double-precision floats (64 bits) as contiguous blocks of 8 bytes each These bytes are what a compression algorithm will see when it tries to compact your data. Byte-shuffling re-arranges your data so that the th byte of each element is stored contiguously:

Below is some idiomatic Python code to implement this concept:

import numpy as np

def byte_shuffle_encode(x: np.array):

byte_buffer = x.view(np.uint8)

return byte_buffer.reshape((-1, x.itemsize)).T.flatten()

def byte_shuffle_decode(byte_buffer: np.array, dtype: np.dtype):

return byte_buffer.reshape((dtype().itemsize, -1)).T.flatten().view(dtype)

When combining this with compression, one would write zstd.compress(byte_shuffle_encode(x)) to encode and byte_shuffle_decode(zstd.decompress(x)) to decode.

## Dictionary compression

Some data arrays are hard to compress simply because their elements span a wide range of possible values but lack structures or patterns that make compression efficient despite many repetitions of the same value, simply because of how they are organized.

A good example of this is ion mobility, where the values span an instrument-specific domain with varying levels of precision, and these points are repeated out-of-order because ion mobility arrays are sorted along the m/z axis.

One aspect of a compression algorithm is to find repeated patterns and encode them in a lookup table or “dictionary”, and associate unique tokens or placeholders that use less space than the original patterns. How big patterns may be, what criteria they must satisfy, and other factors, depend upon the algorithm, but these algorithms generally do not know much about the data they compress, which prevents them from making simplifying assumptions.

Since we know the context in which we are working, we can pre-construct a dictionary with which to encode the data, and apply the general purpose compression algorithm on that encoding instead, assuming it is simpler than the original. At the same time, we recognize that a very convoluted algorithm is far less likely to be implemented, so we aim to make the algorithm simple enough to make it easy to develop additional implementations if the existing portfolio of reference implementations is not sufficient for current needs.

      - Encoding

We construct a lookup table by first collecting all the unique values in a data array , called , and sort  so adjacent values are close together to hopefully make them easier to compress later. Based upon the number of unique items, we can determine the largest bit width “index” or “key” type we need to address them all. For ion mobility data at the time of this writing, there are fewer than 1,000 unique values recorded by instruments today, which fits tidily in 216 bits. Next, we build a mapping from value to its index in  using the smallest standard integer type , and create a new array .

We can then write the following structure in bytes:

We prefix the dictionary encoding with two 8 byte unsigned integers in little endian order. The first is the distance, in bytes, from the beginning of the buffer to the beginning of the region encoding , . The second is the number of unique values in . Next, we encode the bytes of  in the buffer in little endian byte order, followed by the bytes of  in little endian byte order.

      - Decoding

To decode, we first read the first 16 bytes and interpret it as two 64 bit unsigned integers which tells us the offset  to the index list  and the number of values in , . We assume we know the type of value we want to decode  into, so we can validate that  matches the size of that type in bytes. It is then straight forward to reconstruct  from byte 16 to the index offset, reinterpreting the bytes as needed to build a native representation of  in the correct type. Next, we infer the  from  using the same logic as during encoding. Finally, we reconstruct the original data array  using .

The following Python code implements the dictionary encoding and decoding process:

import numpy as np

import struct

bytes_views = {

8: np.uint64,

4: np.uint32,

2: np.uint16,

1: np.uint8,

}

index_widths = [

(2**8, np.uint8),

(2**16, np.uint16),

(2**32, np.uint32),

(2**64, np.uint64),

]

def dict_encode(data: np.typing.ArrayLike) -> bytes:

byte_view_tp = bytes_views[data.itemsize]

view = data.view(byte_view_tp)

values = np.sort(np.unique(view))

index_tp = None

for size, idx_type in index_widths:

if len(values) < size:

index_tp = idx_type

break

else:

raise ValueError(f"Cannot find index dtype for {len(values)} values")

indices = np.zeros(len(view), dtype=index_tp)

for i, k in enumerate(values):

indices[view == k] = i

n_values = np.uint64(len(values))

total_size = 16 + values.nbytes + indices.nbytes

buffer = np.empty(total_size, dtype=np.uint8)

buffer[:8] = memoryview(struct.pack("<Q", np.uint64(16 + values.nbytes)))

buffer[8:16] = memoryview(struct.pack("<Q", np.uint64(len(values))))

buffer[16 : values.nbytes + 16] = values.view(np.uint8)

buffer[(16 + values.nbytes) :] = indices.view(np.uint8)

return buffer

def dict_decode(buffer: np.typing.ArrayLike, dtype: np.dtype) -> bytes:

index_tp = None

byte_view_tp = bytes_views[dtype().itemsize]

(offset_to_data,) = struct.unpack("<Q", buffer[:8])

(n_values,) = struct.unpack("<Q", buffer[8:16])

for size, idx_type in index_widths:

if size > n_values:

index_tp = idx_type

break

else:

raise ValueError(f"Cannot find index dtype for {n_values} indices")

values = buffer[16:offset_to_data].view(byte_view_tp)

indices = buffer[offset_to_data:].view(index_tp)

return values[indices].view(dtype)

      - Combining dictionaries and byte shuffling

When the dictionary is large, and especially when the values are large, it can be advantageous to apply byte shuffling to the values list and the index list separately. This works in two ways. The first is that the sorted value array  is more likely to compress better when shuffled for the same reason byte shuffling works well on an m/z array. The second is that the larger, repetitive index array , being an integer type, is more compressable than a floating point type when shuffled, even unsorted.

This is relatively straight forward to do by injecting the shuffling step immediately prior to encoding the dictionary buffers and decoding them prior to using them as an index map.

The following Python code implements these concepts together:

def dict_encode_with_shuffle(data: np.typing.ArrayLike) -> bytes:

byte_view_tp = bytes_views[data.itemsize]

view = data.view(byte_view_tp)

values = np.sort(np.unique(view))

index_tp = None

for size, idx_type in index_widths:

if len(values) < size:

index_tp = idx_type

break

else:

raise ValueError(f"Cannot find index dtype for {len(values)} values")

indices = np.zeros(len(view), dtype=index_tp)

for i, k in enumerate(values):

indices[view == k] = i

values = byte_shuffle_encode(values)

indices = byte_shuffle_encode(indices)

total_size = 16 + values.nbytes + indices.nbytes

buffer = np.empty(total_size, dtype=np.uint8)

buffer[:8] = memoryview(struct.pack("<Q", np.uint64(16 + values.nbytes)))

buffer[8:16] = memoryview(struct.pack("<Q", np.uint64(len(values))))

buffer[16 : values.nbytes + 16] = values.view(np.uint8)

buffer[(16 + values.nbytes) :] = indices.view(np.uint8)

return buffer

def dict_decode_with_shuffle(buffer: np.typing.ArrayLike, dtype: np.dtype) -> bytes:

index_tp = None

byte_view_tp = bytes_views[dtype().itemsize]

(offset_to_data,) = struct.unpack("<Q", buffer[:8])

(n_values,) = struct.unpack("<Q", buffer[8:16])

for size, idx_type in index_widths:

if size > n_values:

index_tp = idx_type

break

else:

raise ValueError(f"Cannot find index dtype for {n_values} indices")

values = buffer[16:offset_to_data].view(byte_view_tp)

indices = buffer[offset_to_data:].view(index_tp)

indices = byte_shuffle_decode(indices.view(np.uint8), index_tp)

values = byte_shuffle_decode(values.view(np.uint8), byte_view_tp)

return values[indices].view(dtype)

# Authors Information

Eric W. Deutsch

Institute for Systems Biology, Seattle WA, USA

edeutsch@systemsbiology.org

Hans Vissers

Waters Corporation

Stamford Avenue | Altrincham Road, Wilmslow, SK9 4AX, UK

hans_vissers@waters.com

Matt Chambers, Bioinformatics Consultant

Stamford CT, USA

matt.chambers42@gmail.com

Tim Van Den Bossche

Ghent University

Ghent, Belgium

tim.vandenbossche@ugent.be

Tomi Suomi

University of Turku

Turku, Finland

tomi.suomi@utu.fi

Yasset Perez-Riverol

European Bioinformatics Institute

Hinxton, UK

yperez@ebi.ac.uk

Hannes Röst

University of Toronto

160 College Street, Room 604
Toronto, ON M5S 3E1, Canada

hannes.rost@utoronto.ca

Andrew Dowsey

University of Bristol

Oakfield Grove Bristol BS8 2BN, UK

andrew.dowsey@bristol.ac.uk

Pierre-Alain Binz

CHUV Centre Universitaire Hospitalier Vaudois,

CH-1011 Lausanne 14, Switzerland

pierre-alain.binz@chuv.ch

# Contributors

In addition to the authors, a number of additional contributions have been made during the preparation process. The contributors who actively participated to the recommendation documentation are:

Joshua Klein, Boston University

Steffen Neumann, Leibniz Institute of Plant Biochemistry

# Intellectual Property Statement

The PSI takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights. Copies of claims of rights made available for publication and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementers or users of this specification can be obtained from the PSI Chair.

The PSI invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights which may cover technology that may be required to practice this recommendation. Please address the information to the PSI Chair (see contacts information at PSI website).

# Copyright Notice

Copyright (C) Proteomics Standards Initiative (2026). All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works. However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the PSI or other organizations, except as needed for the purpose of developing Proteomics Recommendations in which case the procedures for copyrights defined in the PSI Document process must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by the PSI or its successors or assigns.

This document and the information contained herein is provided on an "AS IS" basis and THE PROTEOMICS STANDARDS INITIATIVE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE."

# Glossary

Not used.

# References

Bhamber, Ranjeet S., Andris Jankevics, Eric W. Deutsch, Andrew R. Jones, and Andrew W. Dowsey. 2021. “mzMLb: A Future-Proof Raw Mass Spectrometry Data Format Based on Standards-Compliant mzML and Optimized for Speed and Storage Requirements.” Journal of Proteome Research 20 (1): 172–83.

Bradner, S. 1997. “Key Words for Use in RFCs to Indicate Requirement Levels.” RFC Editor. https://doi.org/10.17487/rfc2119.

Martens, Lennart, Matthew Chambers, Marc Sturm, Darren Kessner, Fredrik Levander, Jim Shofstahl, Wilfred H. Tang, et al. 2011. “mzML--a Community Standard for Mass Spectrometry Data.” Molecular & Cellular Proteomics: MCP 10 (1): R110.000133.

Mayer, Gerhard, Luisa Montecchi-Palazzi, David Ovelleiro, Andrew R. Jones, Pierre-Alain Binz, Eric W. Deutsch, Matthew Chambers, et al. 2013. “The HUPO Proteomics Standards Initiative- Mass Spectrometry Controlled Vocabulary.” Database: The Journal of Biological Databases and Curation 2013 (March): bat009.

Meier, Florian, Andreas-David Brunner, Max Frank, Annie Ha, Isabell Bludau, Eugenia Voytik, Stephanie Kaspar-Schoenefeld, et al. 2019. “Parallel Accumulation – Serial Fragmentation Combined with Data-Independent Acquisition (diaPASEF): Bottom-up Proteomics with near Optimal Ion Usage.” bioRxiv. bioRxiv. https://doi.org/10.1101/656207.

Moseley, M. Arthur, Christopher J. Hughes, Praveen R. Juvvadi, Erik J. Soderblom, Sarah Lennon, Simon R. Perkins, J. Will Thompson, et al. 2018. “Scanning Quadrupole Data-Independent Acquisition, Part A: Qualitative and Quantitative Characterization.” Journal of Proteome Research 17 (2): 770–79.

Rodriguez-Suarez, Eva, Chris Hughes, Lee Gethings, Kevin Giles, Jason Wildgoose, Martha Stapels, Keith E. Fadgen, et al. 2013. “An Ion Mobility Assisted Data Independent LC-MS Strategy for the Analysis of Complex Biological Samples.” Current Analytical Chemistry 9 (2): 199–211.